Kit 2: Performance of Solar Panels

Introduction

Electricity in Hong Kong is mainly generated by the combustion of fossil fuel. Adoption of renewable energy can help reduce carbon emissions. Solar energy is one of the common renewable energy sources.

It has become more common in recent years to see solar energy generation systems installed at facilities and buildings. This can save on electricity bills and also reduce the use of fossil fuels. By studying the factors affecting the output power of solar panels, students can learn to maximise their efficiency.

This educational kit aims to investigate how the electricity generation of solar panels is affected by different factors through experiments. The scientific knowledge will further help students to explore the performance of solar panels in Kit 4, which involves the analysis of Real-time Monitoring System (RTMS) data from solar panels. This educational kit covers the following topics:

- Effect of Distance on Illuminance

- Effect of Illuminance on Output Power of Solar Panels

- Effect of Incident Angle on Illuminance

- Effect of EM Wavebands & Different Types of Solar Panels

Students are encouraged to watch the video of the experiments and complete the worksheet. The video is available at the QR code below.

Name: __________________________

Kit 2.1: Effect of Distance on Illuminance

Illuminance measures the power of light received by a unit surface area in the unit of lux (lx). A smartphone with a light sensor and the free mobile app “Phyphox” is used to measure illuminance. The LED of another smartphone is used as a light source and is placed on the opposite side at a distance. The two smartphones are carefully aligned so the light source points directly toward the light sensor. Moving one of them along a metre rule changes the distance between the two smartphones. The readings of illuminance are taken at different distances.

- Which of the following factors is NOT kept constant during the experiment?

A. Brightness of the light source
B. Distance between the light source and the light meter
C. Incident angle of light on the light sensor

- In each measurement, a background illuminance is measured with a small opaque cardboard inserted between the light source and the light sensor. What is the purpose of measuring the background illuminance?

The background illuminance is NOT due to light coming directly from the

LED but from ambient light and light reflected from the surroundings.

Direct illuminance due to the LED can be found by subtracting the

background from the total illuminance measured at each distance.

- Record and calculate the data in the following table.

| Distance r (m) | 1/r2 (m-2) | Total illuminance (lx) | Background (lx) |  | Direct illuminance E (lx) |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0.2 | 25.0 | 1100 | - | 13 | = | 1087 |
| 0.3 | 11.1 | 432 | - | 21 | = | 411 |
| 0.4 | 6.25 | 229 | - | 21 | = | 208 |
| 0.6 | 2.78 | 100 | - | 25 | = | 75 |
| 0.8 | 1.56 | 65 | - | 26 | = | 39 |
| 1.0 | 1.00 | 47 | - | 25 | = | 22 |

- Plot a graph of the direct illuminance E versus inverse square of distance 1/r2.

- From the graph in Question 4, what can you conclude about the relationship between the illuminance and distance? Write down the name of the relationship.

The illuminance is inversely proportional to the square of the distance

between the light source and the light meter. The relationship is

known as inverse square law.

- Explain physically why illuminance obeys such a relationship.

Light emitted from a point source travels radially and evenly in all directions.

At a distance r, the light power is evenly distributed on a sphere of surface

area 4πr2. The illuminance, which is equal to the light power divided by the

area, is hence proportional to 1/r2.

- Is the performance of solar panels on Earth’s surface significantly affected by the change in distance between the Earth and the Sun?	Why?

No, the change in distance is very small as the Earth’s orbit is nearly a circle.

Kit 2.2: Effect of Illuminance on Output Power of Solar Panels

A solar panel and a smartphone are placed under an adjustable light source. The smartphone measures illuminance E near the solar panel. The solar panel is connected to a resistor. Two multimeters measure the voltage V across the resistor and the current I through the resistor respectively. Data are taken for different adjusted brightness of the light source. The output power P can be calculated from the formula P = IV.

- Which of the following factors is NOT kept constant during the experiment?

A. Circuit connected to the solar panel
B. Brightness of the light source
C. Distance between the light source and the solar panel
D. Incident angle of light on the light sensor and solar panel

- Record the data in the table below. Take the values to 3 significant figures.

| Illuminance E (lx) | Current I (mA) | Voltage V (V) | Output power P (mW) |
| --- | --- | --- | --- |
| 14600 | 7.64 | 1.54 | 11.8 |
| 30000 | 15.1 | 3.03 | 45.8 |
| 42600 | 20.4 | 4.09 | 83.4 |
| 59500 | 26.8 | 5.37 | 144 |
| 82600 | 31.9 | 6.38 | 204 |
| 113000 | 33.4 | 6.67 | 223 |
| 136000 | 34.0 | 6.79 | 231 |
| 160000 | 34.4 | 6.87 | 236 |

- Plot a graph of output power P against illuminance E.

- How does the output power vary with illuminance at low illuminances?

The output power increases with illuminance.

- How does the output power vary with illuminance at high illuminances?

The output power no longer increases significantly even when the

illuminance is further increased.

- From the experimental results, what inferences could you draw about the effects of weather on the output power of solar panels?

In cloudy weather, the output power of solar panels is low due to low

illuminance. The output power increases with illuminance. In sunny

weather, the illuminance is very high, and the output power would reach

a maximum value.

Kit 2.3: Effect of Incident Angle on Illuminance

This experiment is conducted with two smartphones; one acts as a light source and the other as a light meter, as in Kit 2.1. The smartphones are fixed at the two ends of a metal bar, which allows the light source to be rotated about the light meter at a fixed distance. Initially, they are carefully aligned so that the LED of the light source points directly toward the sensor of the light meter. Rotating the light source changes the angle of incidence θ, and hence the illuminance on the light sensor according to Lambert’s cosine law,

E = E0cosθ,

where E0 is the illuminance when θ = 0, E is the illuminance at angle θ.

- In the table below, record the measured values of illuminance E at different incident angles θ. Calculate the theoretical values of illuminance from the formula E = E0cosθ, taking E0 as the measured value of illuminance at θ = 0. Then, calculate their percentage differences. Give your results correct to 3 significant figures.

| Incident angle θ | Illuminance E (lx) |  | Percentage difference |
| --- | --- | --- | --- |
|  | Measured value | Theoretical value |  |
| 0˚ | 510 | 510 | - |
| 10˚ | 505 | 502 | 0.54% |
| 20˚ | 484 | 479 | 0.93% |
| 30˚ | 450 | 442 | 1.63% |
| 40˚ | 398 | 391 | 1.43% |
| 50˚ | 301 | 328 | 5.26% |
| 60˚ | 111 | 255 | 28.2% |

- Plot the theoretical and measured values of the illuminance against θ.

- What does the graph in Question 2 show at small incident angles?

It shows that the illuminance decreases with the incident angle according to

Lambert’s cosine law up to about θ = 40o.

- Why does the measured illuminance deviate from the theoretical values at large incident angles.

Reflection by the screen of the smartphone may become significant at

large incident angles. The amount of light entering the light sensor could be

much smaller than the theoretical value.

- In real life, what factors may change the incident angle of sunlight on a solar panel over time?

The Sun’s motion in the sky across a day.

The change in Sun’s position in the sky in different seasons.

Kit 2.4: Effect of EM Wavebands & Different Types of Solar Panels

Solar radiation comprises three electromagnetic (EM) wavebands, namely visible light, infrared (IR) and ultraviolet (UV) radiation. This experiment investigates the output powers of a monocrystalline, a polycrystalline, and an amorphous silicon solar panel under these radiations. An IR lamp, a visible light lamp, and a UV lamp are used to illuminate each of the three solar panels in turn. The current I through and voltage V across a resistor are measured by two multimeters. The output power P is then found from the formula P = IV in each case.

- Record the data and calculate the output power of different solar panels under each lamp in the tables. Take the values to 3 significant figures.

| Source: IR | Voltage (V) | Current (mA) | Output Power (mW) |
| --- | --- | --- | --- |
| Monocrystalline | 6.35 | 31.7 | 201 |
| Polycrystalline | 6.06 | 30.3 | 184 |
| Amorphous silicon | 0.073 | 0.35 | 0.026 |

| Source: Visible light | Voltage (V) | Current (mA) | Output Power (mW) |
| --- | --- | --- | --- |
| Monocrystalline | 6.40 | 32.0 | 205 |
| Polycrystalline | 6.28 | 31.4 | 197 |
| Amorphous silicon | 4.38 | 21.9 | 95.9 |

| Source: UV | Voltage (V) | Current (mA) | Output Power (mW) |
| --- | --- | --- | --- |
| Monocrystalline | 1.12 | 5.60 | 6.27 |
| Polycrystalline | 0.760 | 3.78 | 2.87 |
| Amorphous silicon | 0.066 | 0.33 | 0.022 |

- From the results of Question 1, arrange the output power of the 3 types of solar panels in descending order.

| Monocrystalline | > | Polycrystalline | > | Amorphous silicon |
| --- | --- | --- | --- | --- |

- (*Circle the best answer) Although the 3 light sources cannot be compared due to their different powers, it is still observed that solar panels are least responsive to (*IR / visible light / UV). Amorphous silicon solar panels have apparent response to (*IR / visible light / UV) only.