Kit 4: Weather, Seasons & Solar Panels

Introduction

The performance of the solar energy generation systems installed under Solar Harvest is recorded and displayed by the Real-time Monitoring System (RTMS). It gathers long-term data for educational and analytical purposes. We will use some sample data to demonstrate simple scientific analysis.

This educational kit aims to explore the factors affecting the performance of solar energy generation systems and the scientific principles behind. Students will learn how the incident angle of sunlight affects the illuminance on a solar panel. Then, students will be guided to analyse some RTMS data and integrate the knowledge across the kits to understand the performance of solar energy generation systems under different physical conditions. This educational kit covers the following topics:

- Effect of Incident Angle on Illuminance

- Performance of Solar Panels over a Day

- Effect of Weather on the Performance of Solar Panels

- Effect of Seasons on the Performance of Solar Panels

Students are encouraged to watch the video of the experiments and complete the worksheet. The video is available at the QR code below.

Name: __________________________

Kit 4.1: Effect of Angle of Incident Light

This experiment measures the illuminance received by a surface when light is incident at different angles. The LED on a smartphone is used as a light source, another smartphone is used as a light meter to measure the illuminance. The first smartphone (light source) can be rotated about the light sensor of the second one and the angle is measured with a protractor.

- Identify the independent, dependent, and control variables in the experiment by checking the boxes below:

|  | Independent Variables | Dependent Variables | Control Variables |
| --- | --- | --- | --- |
| Brightness of the LED | ☐ | ☐ | ☐ |
| Distance between the LED and the light metre | ☐ | ☐ | ☐ |
| Angle of incident light | ☐ | ☐ | ☐ |
| Illuminance | ☐ | ☐ | ☐ |

- Record the illuminance at different angles of incidence in the following table.

| Angle of Incidence θ | Illuminance (lx) |
| --- | --- |
| 0˚ |  |
| 20˚ |  |
| 40˚ |  |

- Circle the appropriate answers in the statements below.

- At larger angles of incidence, the same amount of light falls onto a (larger / smaller) area. Thus, the illuminance is (higher / lower).

- At larger angles of incidence, the output power of a solar panel (increases / decreases).

Kit 4.2 Performance of Solar Panels over a Day

In this section, the RTMS data recorded on a sunny day are used to observe the variation in power output of solar panels over a day. Both the output power of the solar panels and the solar irradiance are presented in graphs for analysis.

- Circle the best answer in the following statement.

According to the graphs of the RTMS data, the solar irradiance and output power of solar panels in the morning and evening are (higher / lower) than those at noon.

- What is the reason behind the observation in Question 1?

- The Sun emits more sunlight at noon than in the morning and evening.

- The distance between the Sun and the Earth varies over a day.

- The incident angle of sunlight on the solar panels varies over a day.

- The weather at noon is different from that in the morning and evening.

- Why do we compare the performance of solar panels on a sunny day rather than a cloudy day?

- Reduce the effects of clouds and indirect sunlight so that the effect of the change in incident angle can be clearly seen.

- Solar panels do not work on cloudy days.

- Record higher values of output power and solar irradiance for easier comparison.

- Reduce the errors in the values of output power and solar irradiance.

- Which of the following is an adaptive design of solar panels that helps increase the output power over a day?

- Build the solar panels on a spherical surface so that they receive sunlight from all directions.

- Add sun-tracking devices so that the solar panels adjust the angle according to the Sun’s position.

- Add extending arms to raise the solar panels higher so that they can receive more sunlight.

- Install the solar panels on a track so that they can move around and capture more sunlight.

Kit 4.3: Effect of Weather on the Performance of Solar Panels

In this section, the effects of weather on the performance of solar panels are investigated. The RTMS data recorded on a sunny day and a cloudy day are compared. Both the graphs of solar irradiance and output power of the solar panels are presented for analysis.

- Determine whether the following statements are True or False.

| - Solar panels cannot generate electricity on cloudy days. | True / False |
| --- | --- |
| - Cloud and air particles can scatter sunlight to allow solar panels to generate electricity. | True / False |
| - Indirect sunlight reaching the ground gives a higher irradiance than direct sunlight. | True / False |

- Circle the best answers in the following statements.

- On the sunny day, solar irradiance and output power of the solar panels are (higher / lower) during the daytime. The curves are (smoother / rougher).

- On the cloudy day, solar irradiance and output power of the solar panels are (higher / lower) during the daytime. The curves are (smoother / rougher) due to occasional shading from the clouds.

- The sunny and cloudy days selected for comparison are close to each other. Why?

  - To minimize the difference in the Sun’s positions due to seasonal variations

  - To minimize the effects of temperature on the performance of the solar panels

  - To save time for collecting the data

A. I only

B. II only

C. I and II only

D. I, II and III

- Circle the appropriate answer in the following statement.

In general, the average output power of solar panels is (higher / lower) when the average cloud amount is higher in a month.

Kit 4.4: Effect of Seasons on the Performance of Solar Panels

In this section, the effect of seasonal variations on the performance of solar panels is investigated. Computer simulations are used to illustrate the motion of the Earth around the Sun, and the positions of the Sun in the sky as seen by an observer in the Northern Hemisphere in four seasons. RTMS data obtained within a day in summertime and wintertime are compared.

- Circle the appropriate answers in the following statement.

The sunlight shines (more / less) directly near the equator than the higher latitudes. Therefore, solar panels near the equator have (higher / lower) output power than those at high latitudes.

- Match the days in a year with the correct features.

Sunlight shines on the Tropic of Cancer directly. The Northern Hemisphere receives more sunlight.

Sunlight shines on the equator directly. The equator receives more sunlight.

Sunlight shines on the Tropic of Capricorn directly. The Southern Hemisphere receives more sunlight.

Vernal Equinox

Summer Solstice

Autumnal Equinox

Winter Solstice

- In the analysis of RTMS data on a summer day and a winter day, days with similar cloud amount are selected for comparison. Why?

- When comparing RTMS data on the selected days, which of the following factors account for the better performance of solar panels in summer?

- The Earth is closer to the Sun in summer.

- The angle of incidence of sunlight on the solar panels is smaller at the noontime of summer.

- The sunshine duration is longer in summer.

- The weather is better in summer.

- I and II only

- II and III only

- III and IV only

- I and IV only