Kit 4: Seasonal Changes & Solar Energy

Introduction

The performance of the solar energy generation system installed under Solar Harvest is recorded and displayed by the Real-time Monitoring System (RTMS). It gathers long-term data for educational and analytical purposes. We will use some of the sample data to demonstrate simple scientific analysis.

This educational kit aims to equip students with the basic skills to analyse RTMS data. By integrating the knowledge across the kits, students can understand the performance of solar panels under different physical conditions in the longer term. This educational kit covers the following topics:

- Daily & Seasonal Changes

- Solar Spectrum

- Effect of Weather

- Design of Solar Energy Generation Systems

Students are encouraged to watch the video of the experiments and complete the worksheet. The video is available at the QR code below.

Name: __________________________

Kit 4.1: Daily & Seasonal Changes

Lambert’s cosine law has been discussed in Kit 2.3. It is concluded that the larger the incident angle of light, the lower the illuminance on a surface. In the real world, there are two major factors affecting the incident angle of sunlight on the Earth's surface. The first one is the Sun’s motion across the sky over a day. The second one is the variation of the Sun’s positions in the sky in different seasons. Both factors can be observed in properly selected RTMS data.

Part A: The Sun’s Motion over a Day

The graphs of solar irradiance and output power of solar panels on a sunny day are presented in the video.

- Why is it important to select a sunny day for the observation of the effect of the Sun’s motion over a day?

- According to the graph, is the variation of solar irradiance consistent with that of the output power of solar panels? Explain briefly.

- Describe and explain the variations of solar irradiance and output power of solar panels over a day.

Part B: The Sun Paths in Different Seasons

Computer simulations are presented to illustrate the occurrence of seasons, and the change in position of the Sun in the sky in different seasons. The graphs of solar irradiance and output power of solar panels over a day, in summer and winter respectively, are then compared.

- What is the cause of seasons on Earth?

- Circle the appropriate answers in the following statements.

| Summer Solstice | - Direct sunlight on the (Northern Hemisphere / equator / Southern Hemisphere) - The (longest / shortest) sunshine duration in the Northern Hemisphere |
| --- | --- |
| Winter Solstice | - Direct sunlight on the (Northern Hemisphere / equator / Southern Hemisphere) - The (longest / shortest) sunshine duration in the Northern Hemisphere |
| Vernal/Autumnal Equinoxes | - Direct sunlight on the (Northern Hemisphere / equator / Southern Hemisphere) |

- In the analysis of RTMS data, two days with similar cloud amounts are selected for comparison between summer and winter. Why?

- What are the differences between the solar irradiance and output power of solar panels in summer and winter?

- Which season do solar panels perform better? List TWO reasons for the observation.

Kit 4.2: Solar Spectrum

A solar spectrum shows the intensity of EM waves with different wavelengths. In this section, a portable spectrometer is used to measure the spectrum of the Sun and the sky for analysis.

Identify the following spectra and match the spectra with correct answers.

A. The Sun		B. The clear sky

- Explain how you can identify the spectrum of the clear sky in Question 1.

- The solar spectrum taken on the Earth's surface shows lower intensity than that taken in space at different wavelengths. Why?

- Significant absorptions are observed in the infrared range of the spectrum taken on the Earth’s surface. List two gases in the atmosphere which may cause such absorptions. Can you tell the consequence of such absorptions?

Kit 4.3: Effect of Weather

In this section, the effect of weather on the performance of solar panels is discussed. The RTMS data recorded on a sunny day and a cloudy day are compared using the graphs of solar irradiance and output power of the solar panels.

- The selected sunny and cloudy days for comparison are close to each other. Why?

- Why are the curves of the solar irradiance and output power of solar panels irregular on the cloudy day?

- The solar panels can generate electricity even under cloudy weather. Why?

- Plot a graph of the monthly cloud amount and output power of solar panels in 12 months. What is the relationship between them?

- From Kit 4.1, it is learnt that solar panels perform better in summer. In the graph mentioned in Question 4, however, solar panels do not perform best during summer. What is the possible reason?

Kit 4.4: Design of Solar Energy Generation Systems

The incident angle of sunlight has a significant effect on the performance of solar panels. Adaptive designs can be made to maintain a better incident angle of sunlight and improve the efficiency of solar panels. In this section, some of those designs are discussed.

- The latitude of Hong Kong is 22.5˚N. To let the solar panels be exposed to more sunlight in spring and autumn, in which direction and at what angle should we incline the solar panels?

- In practice, the solar panels installed under Solar Harvest are inclined at 14˚ instead of 22.5˚. Which of the following are the reasons for such a design?

  - To receive more sunlight in summer

  - To reduce the force exerted by strong winds on the solar panels

  - To reduce the counterweight used to stabilize the solar panels

  - To prevent adjacent solar panels from casting shadows on each other

- I and II only

- I and III only

- II and IV only

- All of the above

- Which of the following is an adaptive design of solar panels that helps increase the output power over a day?

- Build the solar panels on a spherical surface so that they receive sunlight from all directions.

- Add sun-tracking devices so that the solar panels adjust the angle according to the Sun’s position.

- Add extending arms to raise the solar panels higher so that they are placed closer to the Sun.

- Install the solar panels on a track so that they can move around and capture more sunlight.

- Give an advantage of installing solar panels in remote areas (such as reservoirs).