Kit 4: Seasonal Changes & Solar Energy

Introduction

The performance of the solar energy generation system installed under Solar Harvest is recorded and displayed by the Real-time Monitoring System (RTMS). It gathers long-term data for educational and analytical purposes. We will use some of the sample data to demonstrate simple scientific analysis.

This educational kit aims to equip students with the basic skills to analyse RTMS data. By integrating the knowledge across the kits, students can understand the performance of solar panels under different physical conditions in the longer term. This educational kit covers the following topics:

- Daily & Seasonal Changes

- Solar Spectrum

- Effect of Weather

- Design of Solar Energy Generation Systems

Students are encouraged to watch the video of the experiments and complete the worksheet. The video is available at the QR code below.

Name: __________________________

Kit 4.1: Daily & Seasonal Changes

Lambert’s cosine law has been discussed in Kit 2.3. It is concluded that the larger the incident angle of light, the lower the illuminance on a surface. In the real world, there are two major factors affecting the incident angle of sunlight on the Earth's surface. The first one is the Sun’s motion across the sky over a day. The second one is the variation of the Sun’s positions in the sky in different seasons. Both factors can be observed in properly selected RTMS data.

Part A: The Sun’s Motion over a Day

The graphs of solar irradiance and output power of solar panels on a sunny day are presented in the video.

- Why is it important to select a sunny day for the observation of the effect of the Sun’s motion over a day?

Without the effect of clouds, we can study the effect of the change in the

Sun’s position on the solar irradiance and output power of solar panels over

a day.

- According to the graph, is the variation of solar irradiance consistent with that of the output power of solar panels? Explain briefly.

Yes, the variation of solar irradiance is consistent with that of the output

power of solar panels. When solar irradiance is higher, sunlight provides

more energy for solar panels to generate electricity.

- Describe and explain the variations of solar irradiance and output power of solar panels over a day.

Both the solar irradiance and output power of solar panels are higher at

noon because the incident angles of sunlight are small. They are lower in

the morning and evening because of the larger angles of incidence.

Part B: The Sun Paths in Different Seasons

Computer simulations are presented to illustrate the occurrence of seasons, and the change in position of the Sun in the sky in different seasons. The graphs of solar irradiance and output power of solar panels over a day, in summer and winter respectively, are then compared.

- What is the cause of seasons on Earth?

The rotational axis of the Earth is tilted at 23.5°. It leads to uneven

distribution of sunlight in different regions throughout the year.

- Circle the appropriate answers in the following statements.

| Summer Solstice | - Direct sunlight on the (Northern Hemisphere / equator / Southern Hemisphere) - The (longest / shortest) sunshine duration in the Northern Hemisphere |
| --- | --- |
| Winter Solstice | - Direct sunlight on the (Northern Hemisphere / equator / Southern Hemisphere) - The (longest / shortest) sunshine duration in the Northern Hemisphere |
| Vernal/Autumnal Equinoxes | - Direct sunlight on the (Northern Hemisphere / equator / Southern Hemisphere) |

- In the analysis of RTMS data, two days with similar cloud amounts are selected for comparison between summer and winter. Why?

To reduce the effects of clouds so that seasonal changes can be clearly

observed.

- What are the differences between the solar irradiance and output power of solar panels in summer and winter?

The solar irradiance and output power of solar panels are both higher in

summer and lower in winter.

- Which season do solar panels perform better? List TWO reasons for the observation.

Solar panels perform better in summer because (1) the incident angle of

sunlight is smaller because the Sun is higher in the sky and (2) the sunshine

duration is longer.

Kit 4.2: Solar Spectrum

A solar spectrum shows the intensity of EM waves with different wavelengths. In this section, a portable spectrometer is used to measure the spectrum of the Sun and the sky for analysis.

A

Identify the following spectra and match the spectra with correct answers.

B

A. The Sun		B. The clear sky

- Explain how you can identify the spectrum of the clear sky in Question 1.

Blue light is scattered more by molecules in the Earth’s atmosphere,

resulting in the blue colour of the sky. The spectrum of the clear sky has a

higher proportion of blue light.

- The solar spectrum taken on the Earth's surface shows lower intensity than that taken in space at different wavelengths. Why?

It is due to the absorption and reflection of EM waves by the Earth

atmosphere.

- Significant absorptions are observed in the infrared range of the spectrum taken on the Earth’s surface. List two gases in the atmosphere which may cause such absorptions. Can you tell the consequence of such absorptions?

Carbon dioxide, water vapour. The absorptions of IR cause greenhouse

effect.

Kit 4.3: Effect of Weather

In this section, the effect of weather on the performance of solar panels is discussed. The RTMS data recorded on a sunny day and a cloudy day are compared using the graphs of solar irradiance and output power of the solar panels.

- The selected sunny and cloudy days for comparison are close to each other. Why?

To minimize the effect of seasonal variations on the performance of solar

panels and ensure a fair comparison between two different weather

conditions.

- Why are the curves of the solar irradiance and output power of solar panels irregular on the cloudy day?

The solar irradiance and output power of solar panels are lowered by

occasional cloud cover. They rise again when the clouds leave. Therefore,

the curves are irregular.

- The solar panels can generate electricity even under cloudy weather. Why?

In the presence of clouds, direct sunlight is blocked, but some indirect

sunlight can still reach the solar panels due to scattering and reflection by

the clouds and objects in the environment.

- Plot a graph of the monthly cloud amount and output power of solar panels in 12 months. What is the relationship between them?

The higher the cloud amount, the lower the output power of solar panels,

and vice versa.

- From Kit 4.1, it is learnt that solar panels perform better in summer. In the graph mentioned in Question 4, however, solar panels do not perform best during summer. What is the possible reason?

The effect of weather dominates over the effect of seasons. The weather in

summer may be cloudier and hinder the performance of solar panels.

Kit 4.4: Design of Solar Energy Generation Systems

The incident angle of sunlight has a significant effect on the performance of solar panels. Adaptive designs can be made to maintain a better incident angle of sunlight and improve the efficiency of solar panels. In this section, some of those designs are discussed.

- The latitude of Hong Kong is 22.5˚N. To let the solar panels be exposed to more sunlight in spring and autumn, in which direction and at what angle should we incline the solar panels?

The solar panels should be inclined at 22.5˚ toward South.

- In practice, the solar panels installed under Solar Harvest are inclined at 14˚ instead of 22.5˚. Which of the following are the reasons for such a design?

  - To receive more sunlight in summer

  - To reduce the force exerted by strong winds on the solar panels

  - To reduce the counterweight used to stabilize the solar panels

  - To prevent adjacent solar panels from casting shadows on each other

- I and II only

- I and III only

- II and IV only

- All of the above

- Which of the following is an adaptive design of solar panels that helps increase the output power over a day?

- Build the solar panels on a spherical surface so that they receive sunlight from all directions.

- Add sun-tracking devices so that the solar panels adjust the angle according to the Sun’s position.

- Add extending arms to raise the solar panels higher so that they are placed closer to the Sun.

- Install the solar panels on a track so that they can move around and capture more sunlight.

- Give an advantage of installing solar panels in remote areas (such as reservoirs).

To make better use of land.